S1F1

Command (S1F1) - Request:

S1F1 W

<L [0]>

Response (S1F2)  :

S1F2

<L [3]

<A "EQP-5000">        -- Item 1: MDLN (Model Name)

<A "ETCH-PRO-X100">   -- Item 2: EQPID (Equipment ID)

<A "EXP 2.0">         -- Item 3: SOFTREV (Software Revision)

>

------------------------------------------------------

S1F3

Example: Without Values

Command (S1F3) - Request:

S1F3 W

<L [0]>

Response (S1F4) :

S1F4

<L [80]

<U4 1800>    -- Value for SVID 1

<U4 10>      -- Value for SVID 2

<A "85">     -- Value for SVID 3

...          -- (Total of 80 values)

>

Example: With Values

Command (S1F3) - Request:

S1F3 W

<L [4]

<U4 2001>

<U4 2002>

<U4 2003>

<U4 2006>

>

Response (S1F4):

S1F4

<L [4]

<A "14:55:08">  -- Value for SVID 2001

<A "14:55:08">  -- Value for SVID 2002

<A "66.72224">  -- Value for SVID 2003

<A "84.02369">  -- Value for SVID 2006

>

--------------------------------------------------------------

S1F11

Example: Without Values (Request All)

Command (S1F11) - Request:

S1F11 W

<L [0]>

Response (S1F12) - Standard Format:

S1F12

<L [80]

<L [3]

<U4 1>              -- SVID

<A "Clock">         -- SVNAME

<A "HHMMSS">        -- UNITS

>

<L [3]

<U4 2>

<A "ControlState">

<A "">

>

... (Total of 80 sub-lists)

>

Example: With Values (Specific Request)

Command (S1F11) - Request:

S1F11 W

<L [3]

<U4 2001>

<U4 2002>

<U4 2003>

>

Response (S1F12) - Standard Format:

S1F12

<L [3]

<L [3]

<U4 2001>

<A "ControlState">

<A "">

>

<L [3]

<U4 2002>

<A "ProcessState">

<A "">

>

<L [3]

<U4 2003>

<A "Chamber1Temperature">

<A "C">

>

>

------------------------------------------------------------

S1F13

(Without Values)

Command (S1F13) Request:

S1F13 W

<L [0]>

Typical Response (S1F14):

S1F14

<L [2]

<B [1] 0x00>   -- COMMACK (0 means Accepted)

<L [2]         -- Equipment Info

<A "MODEL_1">

<A "VER_1.0">

>

>

Communications (With Values)

Request :

S1F13 W

<L [2]

<A "HOST_SYS">  -- MDNL: Host Model Name

<A "V2.4.0">    -- SOFTREV: Host Software Version

>

Response :

S1F14

<L [2]

<B 0x00>                  -- COMMACK: 0x00 means "Accepted/Success"

<L [2]

<A "EQP_MODEL_XYZ">     -- MDLN: Equipment Model Name

<A "B6.22.04">          -- SOFTREV: Equipment Software Revision

>

>

------------------------------------------------------

S1F15

Request OFF-LINE (Standard)

Command (S1F15) - Request Direction: Host → Equipment The Host tells the Equipment to go Offline.

Request :

S1F15 W

<L [0]>

Response :

S1F16

<B 0x00>  -- OFLACK: 0x00 means "Acknowledge, Offline"

--------------------------------------------------------

S1F17

This message is the formal request from the Host to the Equipment to change its communication state from OFF-LINE to ON-LINE.

Direction: Host → Equipment Like the offline request, the standard "Online" request is usually sent as an empty list.

Command (S1F17) – Request

S1F17 W

<L [0]>

Response (S1F18) - With Values

S1F18

<B 0x00>  -- ONLACK: 0x00 means "Accepted, now ON-LINE"

-------------------------------------------------------

S1F21

Request ALL Data Variables (Without Values)

Command (S1F21) - Request:

S1F21 W

<L [0]>

Response (S1F22):

S1F22

<L [150]              -- Example: 150 variables found

<L [3]

<U4 1001>         -- DVID

<A "Temperature"> -- DVNAME

<A "Celsius">     -- UNITS

>

<L [3]

<U4 1002>         -- DVID

<A "Pressure">    -- DVNAME

<A "kPa">         -- UNITS

>

...

>

Request SPECIFIC Data Variables (With Values)

Command (S1F21) - Request:

S1F21 W

<L [2]

<U4 501>            -- DVID for Motor Speed

<U4 505>            -- DVID for Gas Flow

>

Response (S1F22):

S1F22

<L [2]

<L [3]

<U4 501>

<A "MotorSpeed">

<A "RPM">

>

<L [3]

<U4 505>

<A "GasFlow Rate">

<A "Ltr/min">

>

>

-----------------------------------------------------

S1F23

With S1F23, you are now performing the Collection Event Namelist Request

Request ALL Collection Events (Without Values)

Command (S1F23) - Request:

S1F23 W

<L [0]>

Response (S1F24):

S1F24

<L [50]               -- Example: 50 events found

<L [3]

<U4 5001>         -- CEID

<A "ProcessStart">-- CENAME

<L [2]            -- Related DVIDs

<U4 1001>

<U4 1002>

>

>

...

>

Request SPECIFIC Collection Events (With Values)

If you only need the definition for specific Event IDs, you list those CEIDs in the request.

Command (S1F23) - Request:

S1F23 W

<L [2]

<U4 100>            -- CEID for "Door Open"

<U4 200>            -- CEID for "Cycle Complete"

>

Response (S1F24):

S1F24

<L [2]

<L [3]

<U4 100>

<A "DoorOpen">

<L [0]>           -- No specific DVIDs tied to this

>

<L [3]

<U4 200>

<A "CycleComplete">

<L [1]

<U4 3005>       -- DVID for Units Processed

>

>

>

----------------------------------------------------------

S2F13

1. Request ALL Equipment Constants (Without Values)

If you send an empty list, the equipment returns the current value of every configuration constant it has.

Command (S2F13) - Request:

S2F13 W

<L [0]>

Response (S2F14):

S2F14

<L [120]              -- Total of 120 Constants

<U4 600>            -- Value for ECID 1

<A "ENABLED">       -- Value for ECID 2

<F4 25.5>           -- Value for ECID 3

...

>

Request SPECIFIC Equipment Constants (With Values)

If you only need to check specific settings, you list the ECIDs in the request.

Command (S2F13) - Request:

S2F13 W

<L [3]

<U4 105>            -- ECID for Baud Rate

<U4 210>            -- ECID for Over-Temp Limit

<U4 305>            -- ECID for Robot Speed %

>

Response (S2F14):

S2F14

<L [3]

<U4 9600>           -- Value for ECID 105

<F4 150.0>          -- Value for ECID 210

<U4 80>             -- Value for ECID 305

>

-----------------------------------------------------------

S2F15

While S2F13 was for reading settings, S2F15 is for writing or changing them. This is how the Host remotely reconfigures the equipment (e.g., changing a timeout value, a temperature setpoint, or a speed limit).

New Equipment Constant Send (Without Values)

Technically, according to the SECS-II standard, sending an empty list for S2F15 is generally not used because there is nothing for the machine to change. However, if sent, it would look like this:

Command (S2F15) - Request:

S2F15 W

<L [0]>

Response (S2F16):

S2F16

<B 0x00>  -- EAC: 0x00 means "Accepted/No Error"

New Equipment Constant Send (With Values)

Command (S2F15) - Request:

S2F15 W

<L [2]

<L [2]

<U4 210>        -- ECID: Over-Temp Limit

<F4 155.0>      -- NEW VALUE: Change to 155.0

>

<L [2]

<U4 305>        -- ECID: Robot Speed %

<U4 90>         -- NEW VALUE: Change to 90

>

>

Response (S2F16):

S2F16

<B 0x00>  -- EAC: 0x00 means "All constants changed successfully"

-----------------------------------------------------------------

S2F17

With S2F17, you are now performing the Date and Time Request.

This is a very common command used by the Host to synchronize its clock with the Equipment. In a SECS/GEM environment, having matching timestamps is critical for log analysis and troubleshooting.

Request Date and Time (Without Values)

Command (S2F17) - Request:

S2F17 W

<L [0]>

Response (S2F18):

The response returns the current time from the Equipment. The format is almost always a 12-character or 14-character string (YYMMDDhhmmss).

S2F18

<A "260221102837">  -- Year: 26, Month: 02, Day: 21, Time: 10:28:37

Request Date and Time (With Values)

While S2F17 is almost always an empty list, some equipment drivers or specific implementations allow you to request the time in a list format, though this is rare.

Command (S2F17) - Request:

S2F17 W

<L [0]>

Response (S2F18) - With Values:

S2F18

<A "2026-02-21T10:28:37.500"> -- Extended ISO format with milliseconds

-------------------------------------------------------------

S2F23

This is one of the most powerful data collection commands in SECS/GEM. Instead of waiting for an event to happen (like a "Process Start"), a Trace tells the equipment to "snap" data automatically at a specific time interval (e.g., every 5 seconds).

Trace Initialize Send (Without Values)

While you can technically send an empty list, it is rarely done because a Trace requires specific parameters to function

Command (S2F23) - Request:

S2F23 W

<L [0]>

Response (S2F24):

S2F24

<B 0x01>  -- TIAACK: 0x01 usually means "Error/Invalid Parameters" (if the list was empty)

Trace Initialize Send (With Values)

Command (S2F23) - Request:

S2F23 W

<L [5]

<U4 101>            -- TRID: Trace Request ID (Unique ID for this trace)

<A "000010">        -- DSPER: Data Sampling Period (HHMMSS format - 10 seconds)

<U4 100>            -- TOTSMP: Total Samples (How many times to collect)

<U4 1>              -- REPGSZ: Reporting Group Size (Send data every 1 sample)

<L [2]              -- List of SVIDs to collect

<U4 1001>         -- SVID for Temperature

<U4 1002>         -- SVID for Pressure

>

>

Response (S2F24):

S2F24

<B 0x00>  -- TIAACK: 0x00 means "Trace Started Successfully"

-----------------------------------------------------------

S2F29

This message is used to "discover" the definitions of the Equipment Constants (ECIDs). While S2F13 gives you the values, S2F29 gives you the metadata—like the name of the constant, the minimum and maximum allowed values, and the default setting.

Request ALL ECID Definitions (Without Values)

Command (S2F29) - Request:

S2F29 W

<L [0]>

Response (S2F30):

The response is a list of descriptions for every ECID.

S2F30

<L [100]              -- Example: 100 constants defined

<L [6]

<U4 105>          -- ECID

<A "BaudRate">    -- ECNAME

<U4 1200>         -- ECMIN (Minimum value)

<U4 19200>        -- ECMAX (Maximum value)

<U4 9600>         -- ECDEF (Default value)

<A "bps">         -- UNITS

>

...

>

Request SPECIFIC ECID Definitions (With Values)

Command (S2F29) - Request:

S2F29 W

<L [2]

<U4 210>            -- ECID for Temperature Limit

<U4 305>            -- ECID for Robot Speed

>

Response (S2F30):

S2F30

<L [2]

<L [6]

<U4 210>

<A "OverTempLimit">

<F4 0.0>          -- Min

<F4 200.0>        -- Max

<F4 150.0>        -- Default

<A "Celsius">

>

<L [6]

<U4 305>

<A "RobotSpeed">

<U4 10>           -- Min

<U4 100>          -- Max

<U4 50>           -- Default

<A "%">

>

>

-----------------------------------------------------------

S2F31

Date and Time Set (Without Values)

Command (S2F31) - Request:

S2F31 W

<A "">  -- Empty string (usually causes an error)

Response (S2F32):

S2F32

<B 0x01>  -- TIACK: 0x01 means "Time change rejected/Error"

Date and Time Set (With Values)

The Host sends the current timestamp. The format must match what the equipment expects (typically 12 or 16 characters).

Command (S2F31) - Request:

S2F31 W

<A "260221104515">  -- Set to: Feb 21, 2026, 10:45:15

Response (S2F32):

S2F32

<B 0x00>  -- TIACK: 0x00 means "Time set successfully"

-----------------------------------------------------------------------

S2F33

In SECS/GEM, a Report is a bucket of data variables (DVIDs). You use S2F33 to tell the equipment, "I want to create a Report with ID #10, and inside that report, I want you to include Temperature, Pressure, and Speed.

Delete All Reports (Without Values)

Command (S2F33) - Request:

S2F33 W

<L [2]

<U4 0>          -- DATAID: 0 means we are clearing/defining

<L [0]>         -- Empty list means "Delete all existing reports"

>

Response (S2F34):

S2F34

<B 0x00>  -- DRACK: 0x00 means "Accepted/Reports Cleared"

Define Specific Reports (With Values)

Here, you define which Data Variables (DVIDs) belong to a specific Report ID (RPTID).

Command (S2F33) - Request:

S2F33 W

<L [2]

<U4 1>          -- DATAID: Tracking ID for this message

<L [1]          -- We are defining 1 report in this message

<L [2]

<U4 10>     -- RPTID: The ID for this new Report

<L [3]      -- List of 3 DVIDs to include in this report

<U4 1001> -- DVID for Temperature

<U4 1002> -- DVID for Pressure

<U4 1005> -- DVID for Recipe Name

>

>

>

>

Response (S2F34):

S2F34

<B 0x00>  -- DRACK: 0x00 means "Report Defined Successfully"

-------------------------------------------------------------------

S2F35

Command (S2F35) - Request:

S2F35 W

<L [2]

<U4 0>          -- DATAID: 0 means we are modifying links

<L [0]>         -- Empty list means "Unlink all reports from all events"

>

Response (S2F36):

S2F36

<B 0x00>  -- LRACK: 0x00 means "Accepted/Links Cleared"

Link Report to Event (With Values)

This command connects a CEID (The event trigger) to one or more RPTIDs (The data reports).

Command (S2F35) - Request:

S2F35 W

<L [2]

<U4 1>              -- DATAID

<L [1]              -- Number of Events we are linking

<L [2]

<U4 5001>       -- CEID: "Process End" event

<L [1]          -- List of Report IDs to send when this event hits

<U4 10>       -- RPTID: Send Report #10

>

>

>

>

Response (S2F36):

S2F36

<B 0x00>  -- LRACK: 0x00 means "Link Successful"

------------------------------------------------------------------------

S2F37

Disable All Events (Without Values)

It is common to disable everything before starting a new session to ensure you only get the data you specifically asked for.

Command (S2F37) - Request:

S2F37 W

<L [2]

<B 0x00>        -- CEED: 0x00 means "Disable"

<L [0]>         -- Empty list means "Apply to ALL events"

>

Response (S2F38):

S2F38

<B 0x00>  -- ERACK: 0x00 means "All events disabled"

Enable Specific Events (With Values)

Command (S2F37) - Request:

S2F37 W

<L [2]

<B 0x01>            -- CEED: 0x01 means "Enable"

<L [1]              -- Number of events to enable

<U4 5001>         -- CEID: The ID of the event to turn on

>

>

Response (S2F38):

S2F38

<B 0x00>  -- ERACK: 0x00 means "Event enabled successfully"

-----------------------------------------------------------------

S2F41

Host Command Send (Without Parameters)

Command (S2F41) - Request:

S2F41 W

<L [2]

<A "STOP">      -- RCMD: The Remote Command Name

<L [0]>         -- No parameters required

>

Response (S2F42):

S2F42

<B 0x00>  -- HCACK: 0x00 means "Command Accepted/Executing"

Host Command Send (With Parameters)

Command (S2F41) - Request:

S2F41 W

<L [2]

<A "START">         -- RCMD: The Remote Command

<L [2]              -- Two parameters provided

<L [2]

<A "RECIPE">    -- CPNAME: Command Parameter Name

<A "GOLD_B_22"> -- CPVAL: The actual Recipe Name

>

<L [2]

<A "LOT_ID">    -- CPNAME: Command Parameter Name

<A "BATCH_99">  -- CPVAL: The actual Lot ID

>

>

>

Response (S2F42):

S2F42

<B 0x00>  -- HCACK: 0x00 means "Command Accepted and Started"

-----------------------------------------------------------------

S2F43

Spooling is a "buffer" feature in SECS/GEM. If the communication between the Host and Equipment breaks, the Equipment saves (spools) important messages in its memory. When the connection is restored, the Host uses S2F43 to clear out that buffer or reset the spooling state.

Reset Spooling (Without Values/Select All)

Command (S2F43) - Request:

S2F43 W

<L [0]>

Response (S2F44):

S2F44

<B 0x00>  -- RSACK: 0x00 means "Reset Successful"

Reset Spooling (With Values/Specific Streams)

Command (S2F43) - Request:

S2F43 W

<L [1]

<L [2]

<U2 6>    -- STRID: Stream 6 (Data Collection)

<L [0]>   -- FCNID: Empty list here means "All functions in Stream 6"

>

>

Response (S2F44):

S2F44

<B 0x00>  -- RSACK: 0x00 means "Reset Successful"

-------------------------------------------------------------

S2F47

While S2F43 was used to reset or clear the spool, S2F47 is used to turn spooling on or off and to define how the spooling buffer should behave (for example, whether to overwrite old data when the buffer is full).

Spooling Management (Without Values / Inquiry)

Command (S2F47) - Request:

S2F47 W

<L [0]>

Response (S2F48):

S2F48

<B 0x01>  -- STRACK: 0x01 means "Invalid/Denied" (because specific parameters are missing)

Spooling Management (With Values / Enable)

Command (S2F47) - Request:

S2F47 W

<L [2]

<B 0x01>            -- SPCNT: 0x01 means "Enable Spooling"

<L [1]              -- Number of Stream/Functions to spool

<L [2]

<U2 6>          -- STRID: Stream 6 (Event Reports)

<L [0]>         -- FCNID: All functions in Stream 6

>

>

>

Response (S2F48):

S2F48

<B 0x00>  -- STRACK: 0x00 means "Spooling transition accepted"

---------------------------------------------------------------------------

S2F49

This is the modern, more flexible version of S2F41. While the older S2F41 was limited in how it handled complex data, S2F49 allows the Host to send commands with much more detail, specifically using Object-Oriented structures. This is commonly used for complex tasks like "Carrier Loading" or "Process Job" starts.

Enhanced Remote Command (Simple/No Parameters)

Command (S2F49) - Request:

S2F49 W

<L [4]

<A "PAUSE">     -- RCMD: The Remote Command

<L [0]>         -- OBJSPEC: No specific object target

<L [0]>         -- OBJTYPE: No specific object type

<L [0]>         -- CP: No parameters

>

Response (S2F50):

S2F50

<L [2]

<B 0x00>        -- HCACK: 0x00 means "Accepted"

<L [0]>         -- ERRORS: No parameter errors

>

Enhanced Remote Command (With Values/Parameters)

Command (S2F49) - Request:

S2F49 W

<L [4]

<A "START">         -- RCMD: The Command

<A "PORT_01">       -- OBJSPEC: The specific target (e.g., Load Port 1)

<A "LOADPORT">      -- OBJTYPE: What kind of object it is

<L [1]              -- Parameters for this command

<L [2]

<A "RECIPE_ID"> -- CPNAME

<A "FAST_ETCH"> -- CPVAL

>

>

>

Response (S2F50):

S2F50

<L [2]

<B 0x00>            -- HCACK: 0x00 means "Accepted"

<L [0]>             -- No errors

>

--------------------------------------------------------------------

S5F1

Unlike the previous commands, this message is almost always Equipment-initiated. This means the machine sends it to the Host immediately whenever a hardware problem, safety violation, or process error occurs.

Alarm Report (Without Values / Clear)

Command (S5F1) – Request

S5F1 W

<L [3]

<B 0x00>        -- ALCD: Alarm Code (Bit 8 is 0, meaning Alarm Cleared)

<U4 1024>       -- ALID: Alarm ID (e.g., Motor Jam)

<A "Motor Jam"> -- ALTX: Alarm Text

>

Response (S5F2)

S5F2

<B 0x00>          -- ACKC5: 0x00 means "Alarm Received"

Alarm Report (With Values / Set)

Command (S5F1) – Request

S5F1 W

<L [3]

<B 0x81>        -- ALCD: 0x81 (Bit 8 is 1, meaning Alarm SET/Occurred)

<U4 2050>       -- ALID: Alarm ID (e.g., Over Temperature)

<A "Chamber 1 Over Temp"> -- ALTX: Descriptive Text

>

Response (S5F2)

S5F2

<B 0x00>          -- ACKC5: Host acknowledges the error

-------------------------------------------------------------------------

S5F3

S5F1 is the message that actually sends the alarm, S5F3 is the control mechanism. The Host uses this to tell the equipment which alarms it actually wants to hear about. This prevents the Host from being "spammed" by minor or irrelevant warnings.

Enable/Disable ALL Alarms (Without Values)

Command (S5F3) - Request:

S5F3 W

<L [2]

<B 0x80>        -- ALED: 0x80 means "Enable" (0x00 would be Disable)

<L [0]>         -- ALID: Empty list means "All Alarms"

>

Response (S5F4):

S5F4

<B 0x00>  -- ACKC5: 0x00 means "Accepted/All alarms toggled"

Enable/Disable SPECIFIC Alarms (With Values)

Command (S5F3) - Request:

S5F3 W

<L [2]

<B 0x00>            -- ALED: 0x00 (Disable)

<L [2]              -- List of 2 Alarms

<U4 1024>         -- ALID: "Motor Jam"

<U4 2050>         -- ALID: "Over Temp"

>

>

Response (S5F4):

S5F4

<B 0x00>  -- ACKC5: 0x00 means "Accepted"

-------------------------------------------------------------------

S5F5

Request ALL Alarms (Without Values)

If you send an empty list, the equipment will return the status and description of every alarm it has defined.

Command (S5F5) - Request:

S5F5 W

<L [0]>

Response (S5F6):

S5F6

<L [75]               -- Example: 75 alarms total

<L [3]

<B 0x81>          -- ALCD (Bit 8=1: Currently SET/Active)

<U4 1024>         -- ALID

<A "Motor Jam">   -- ALTX

>

<L [3]

<B 0x01>          -- ALCD (Bit 8=0: Currently CLEAR/Inactive)

<U4 2050>         -- ALID

<A "Over Temp">   -- ALTX

>

...

>

Request SPECIFIC Alarms (With Values)

Command (S5F5) - Request:

S5F5 W

<L [2]

<U4 500>            -- Check Gas Leak Alarm

<U4 501>            -- Check Vacuum Loss Alarm

>

Response (S5F6):

S5F6

<L [2]

<L [3]

<B 0x01>          -- Status of 500 (Clear)

<U4 500>

<A "Gas Leak">

>

<L [3]

<B 0x01>          -- Status of 501 (Clear)

<U4 501>

<A "Vacuum Loss">

>

>

-------------------------------------------------------------------

S5F7

Request Enabled Alarms (Without Values)

By standard, you send an empty list to ask the machine to show everything it is currently "allowed" to report to the Host.

Command (S5F7) - Request:

S5F7 W

<L [0]>

Response (S5F8):

S5F8

<L [15]               -- Only 15 alarms are currently enabled

<L [3]

<B 0x81>          -- ALCD (Category + Set/Clear status)

<U4 1024>         -- ALID

<A "Emergency Stop"> -- ALTX

>

<L [3]

<B 0x01>          -- ALCD

<U4 2050>         -- ALID

<A "Chamber Pressure Low"> -- ALTX

>

...

>

--------------------------------------------------------------------------------------

S6F15

Request Specific Event Report (With Values)

Command (S6F15) - Request:

S6F15 W

<U4 5001>         -- CEID: The Event ID you are polling

Response (S6F16):

S6F16

<L [2]

<U4 1>              -- DATAID

<L [1]              -- List of Reports linked to this CEID

<L [2]

<U4 10>         -- RPTID: Report ID #10

<L [3]          -- The Values

<A "RECIPE_A">

<F4 22.5>

<U4 1200>

>

>

>

>

Handling Errors (Empty Response)

Response (S6F16) - Error/Empty:

S6F16

<L [0]>   -- No data found for that CEID

-----------------------------------------------------------------------

S6F19

S6F19 asks for data based on the Report ID (RPTID) itself. This is useful if you want to check the current values of a specific group of variables you defined earlier in S2F33 without waiting for any machine trigger.

Request a Single Report (With Values)

Command (S6F19) - Request:

S6F19 W

<U4 10>           -- RPTID: The Report ID you want to pull

Response (S6F20):

S6F20

<L [n]            -- List of variables in the report

<A "RUNNING">   -- Value of first SVID (e.g., Status)

<F4 98.2>       -- Value of second SVID (e.g., Temp)

<U4 550>        -- Value of third SVID (e.g., Pressure)

>

Requesting a Non-Existent Report

If the Host requests an RPTID that hasn't been defined yet, the equipment returns a zero-length list.

Response (S6F20) - Error:

S6F20

<L [0]>           -- Report ID not found or not defined

----------------------------------------------------------------------

S6F21

Annotated Event Report (Equipment to Host)

Command (S6F21) - Request:

S6F21 W

<L [3]

<U4 1>              -- DATAID

<L [2]              -- Event Information

<U4 5001>         -- CEID (Event ID)

<A "ProcessEnd">  -- CENAME (The actual name of the event)

>

<L [1]              -- List of Reports

<L [2]

<U4 10>         -- RPTID

<L [1]          -- List of Annotated Variables

<L [3]

<U4 1001>   -- VID (Variable ID)

<A "Temp">  -- VNAME (Variable Name)

<F4 98.6>   -- V (The actual value)

>

>

>

>

>

Response (S6F22):

S6F22

<B 0x00>  -- ACKC6: Host acknowledges receipt

----------------------------------------------------------------------

S6F23

Request Spooled Data command.

Request All Spooled Data (With Values)

Command (S6F23) - Request:

S6F23 W

<U1 0x01>  -- RSCID: 0x01 means "Transmit all spooled messages"

Response (S6F24):

S6F24

<L [3]

<B 0x00>        -- RSDA: 0x00 means "Request Accepted, starting transmission"

<L [0]>         -- Any Stream/Function that could NOT be spooled (empty if all good)

<U4 150>        -- TOTSMP: Total number of messages currently in the spool

>

---------------------------------------------------------------------------

S7F1

Process Program Load Inquire.

Process Program Load Inquire (With Values)

Command (S7F1) - Request:

S7F1 W

<L [2]

<A "GOLD_ETCH_V2">  -- PPID: The name of the recipe

<U4 15240>          -- LENGTH: Size of the recipe in bytes

>

Response (S7F2):

S7F2

<B 0x00>  -- PPGNT: 0x00 means "Permission Granted/Ready"

Handling Permission Denied

Response (S7F2) - Denied:

S7F2

<B 0x02>  -- PPGNT: 0x02 means "No storage space available"

--------------------------------------------------------------------------

S7F3

Process Program Send.

This message follows a successful S7F1 handshake. It contains the actual "recipe" data—the specific instructions, parameters, and settings the equipment needs to process a wafer, chip, or part.

Process Program Send (The Data Upload)

Command (S7F3) - Request:

S7F3 W

<L [2]

<A "GOLD_ETCH_V2">      -- PPID: The Recipe Name

<B 0x47 0x4F 0x4C 0x44...> -- PPBODY: The actual recipe instructions (Binary)

>

Response (S7F4):

S7F4

<B 0x00>  -- ACKC7: 0x00 means "Recipe Received and Saved Successfully"

Handling Errors in Recipe Data

Response (S7F4) - Error:

S7F4

<B 0x03>  -- ACKC7: 0x03 means "Illegal Data" (Format or content is wrong)

-----------------------------------------------------------------

S7F5

Process Program Request.

S7F3 was used to upload a recipe from the Host to the machine, S7F5 is the reverse—it is a "Download" or "Upload to Host" request. The Host uses this to ask the machine for the actual content of a recipe stored in its local memory. This is essential for backing up recipes or verifying that the machine's local version hasn't been modified by an operator.

Request Recipe Content (With Values

The Host sends the PPID (Process Program ID) of the recipe it wants to retrieve.

Command (S7F5) - Request:

S7F5 W

<A "GOLD_ETCH_V2">  -- PPID: The name of the recipe you want to "read"

Response (S7F6):

S7F6

<L [2]

<A "GOLD_ETCH_V2">      -- PPID

<B 0x47 0x4F 0x4C 0x44...> -- PPBODY: The raw recipe data

>

Requesting a Non-Existent Recipe

Response (S7F6) - Error:

S7F6

<L [0]>  -- Recipe not found

----------------------------------------------------------------------------

S7F17

Delete Process Program Send.

Delete Specific Recipes (With Values)

Command (S7F17) - Request:

S7F17 W

<L [n]                -- List of recipes to delete

<A "OLD_RECIPE_V1">

<A "TEST_RECIPE_01">

>

Response (S7F18):

S7F18

<B 0x00>  -- ACKC7: 0x00 means "All specified recipes deleted"

Delete ALL Recipes (Without Values)

Command (S7F17) - Request:

S7F17 W

<L [0]>   -- Delete EVERYTHING

Response (S7F18):

S7F18

<B 0x00>  -- ACKC7: 0x00 means "Storage cleared successfully"

-----------------------------------------------------------------------

S7F19

Current EPPD Request

Request Entire Recipe Directory (Without Values)

By standard, you send this as an empty request. The machine is expected to reply with every recipe name it has saved.

Command (S7F19) - Request:

S7F19 W

<L [0]>

Response (S7F20):

S7F20

<L [4]                -- The machine has 4 recipes

<A "GOLD_ETCH_V2">

<A "SILVER_ETCH_V1">

<A "CLEAN_CYCLE_01">

<A "TEST_WAFER_PARAM">

>

Handling an Empty Directory

Response (S7F20) - Empty:

S7F20

<L [0]>   -- No recipes found in memory

------------------------------------------------------------------------------

S10F3

Terminal Display

Terminal Display (With Values)

Command (S10F3) - Request:

S10F3 W

<L [2]

<B 0x00>                -- TID: Terminal ID (0x00 is usually the main console)

<L [1]                  -- List of text lines

<A "LOAD LOT #1234">  -- TEXT: The message for the operator

>

>

Response (S10F4):

S10F4

<B 0x00>  -- ACKC10: 0x00 means "Message displayed successfully"

Multi-Line Display

Command (S10F3) - Request:

S10F3 W

<L [2]

<B 0x00>

<L [2]

<A "RECIPE DOWNLOAD COMPLETE">

<A "PRESS START TO BEGIN">

>

>

-----------------------------------------------------